# Solution to Charge Pump Differential Equations

Silicon DSP Corporation

http://www.silicondsp.com

Author: Sasan Ardalan

© 2007-2017 Sasan Ardalan

Copyright (C)  2007-2018 Sasan Ardalan

Permission is granted to copy, distribute and/or modify this document

under the terms of the GNU Free Documentation License, Version 1.3

or any later version published by the Free Software Foundation;

with no Invariant Sections, no Front-Cover Texts, and no Back-Cover Texts.

A copy of the license is included in the section entitled "GNU

Free Documentation License".

## Summary:

A charge pump “C” Code Block for Capsim simulation has been written based on the

the formulas for the solution of the charge pump equations presented in Hanumolu, et. al [1]. The results show excellent correspondence with  a Spice model. The differential equations were solved using State Space methods in [1]  and accurately capture the initial conditions.  A Capsim block has been developed called CHPStateSpace based on the approach in [1]. It has been successfully used to model a charge pump PLL. See separate documentation (http://www.silicondsp.com). Note the Capsim High Level Model of the PLL can be used prior to Spice simulations to speed up design and optimization.

## Results:

Figure 1 shows the results for R=1kOhms, C1=2pf and C2=0.2pf. To show the impact of C2 in smoothing the ripples Figure shows the case with C2=2pf and C1=2pf. Note this is for illustration. We need C1 >> C2.

Figure 1 Charge Pump During Up Cycle

Appendix A shows the formulas from [1]. The C code is presented in Appendix B. Note tp is the duration of the “Up” pulse and T_ is the duration for example of the period of the reference or next edge of VCO/N.    Ip is the current source value.

Figure 2 v(1) with C1=2 pf and C2=0.2 pf

Figure 3 v(1) with C1=2 pf and C2=2 pf

## References

[1] Hanumolu, P.K.; Brownlee, M.; Mayaram, K.; Un-Ku Moon,“Analysis of charge-pump phase-locked loops”, Circuits and Systems I: Regular Papers, IEEE Transactions on Circuits and Systems I: Fundamental Theory and Applications, Volume 51,  Issue 9,  Date: Sept. 2004,  Pages:  1665 – 1674

## Appendix A

In the following from [1], vctrl is v1. and vc is v2 across C2. tp is the on time of Up or Down pulse. T_ is related to the period of the reference or the next edge of VCO/N. That is  the time tp<t<T_ is the off time of the Up or Down pulse. See Figure 3.

Figure 3 Definition of tp ad T_ from [1].

In the above note that all the exponentials are based on e-at, where, as we derived before:

## Appendix B:  C Code.

#include <math.h>

#include <stdio.h>

int main(int argc, char* argv[])

{

double c1=2e-12;

double c2=2e-12;

double r=1000;

double Ip= 0.00001;

double dt=0.1e-9;

double tmax=100e-9;

double t;

double tt;

double tdelay=10e-9;

double a;

double v1;

double invC2;

double aInv;

double coeff;

double beta;

double vctrl;

double vctrlInit=0;

double vc;

double vcInit=0;

double tp=10e-9;

double T_;

double g1;

double g2;

double w2;

double wz;

int i;

int n=100;

int flag=0;

FILE *fp;

fp=fopen("chargepump.dat","w");

if(!fp) {

fprintf(stderr,"Could not open file to write:chargepump.dat \n");

return 0;

}

tp=10e-9;

T_=40e-9;

tmax=T_;

dt=tmax/(double)n;

a=(c1+c2)/(r*c1*c2);

aInv = 1/a;

w2=1.0/(r*c2);

wz=1.0/(r*c1);

printf("a=%le\taInv=%le\tw2=%le\n",a,aInv,w2);

vctrlInit=0;

vcInit=0;

flag=0;

for(i=0; i<n+1; i++) {

t=i*dt;

if(t <= tp) {

g1=exp(-a*t);

g2=aInv*(1-exp(-a*t));

vctrl = vctrlInit*(g1+wz*g2)+

vcInit*w2*g2+

(Ip/c2)*(g2+wz*(g1-1)/(a*a)+wz*t/a);

vc=vctrlInit*wz*g2+vcInit*(g1+w2*g2)+

(Ip/c2)*(wz*(g1-1)/(a*a)+wz*t/a);

}

if ((t > tp) && (t<T_) && !flag) {

vctrlInit=vctrl;

vcInit=vc;

flag=1;

}

if ((t > tp) && (t<T_) && flag) {

tt=t-tp;

g1=exp(-a*tt);

g2=aInv*(1-exp(-a*tt));

vctrl=vctrlInit*(g1+wz*g2)+vcInit*w2*g2;

vc=vctrlInit*wz*g2+vcInit*(g1+w2*g2);

}

//printf("%le\t%le\n",t,vctrl);

printf("%le\t%le\t%le\t%le\t%le\t%d\n",t,vctrl,vc,g1,g2,flag);

fprintf(fp,"%le\t%le\n",t,vctrl);

}

fclose(fp);

return 0;

}

# Appendix C

The file: chargepump.dat

0.000000e+00	0.000000e+00

4.000000e-10	1.824200e-03

8.000000e-10	3.376678e-03

1.200000e-09	4.747014e-03

1.600000e-09	5.995259e-03

2.000000e-09	7.161662e-03

2.400000e-09	8.273205e-03

2.800000e-09	9.347975e-03

3.200000e-09	1.039809e-02

3.600000e-09	1.143169e-02

4.000000e-09	1.245421e-02

4.400000e-09	1.346931e-02

4.800000e-09	1.447943e-02

5.200000e-09	1.548621e-02

5.600000e-09	1.649076e-02

6.000000e-09	1.749380e-02

6.400000e-09	1.849585e-02

6.800000e-09	1.949722e-02

7.200000e-09	2.049813e-02

7.600000e-09	2.149875e-02

8.000000e-09	2.249916e-02

8.400000e-09	2.349944e-02

8.800000e-09	2.449962e-02

9.200000e-09	2.549975e-02

9.600000e-09	2.649983e-02

1.000000e-08	2.749989e-02

1.040000e-08	2.667572e-02

1.080000e-08	2.612327e-02

1.120000e-08	2.575295e-02

1.160000e-08	2.550472e-02

1.200000e-08	2.533832e-02

1.240000e-08	2.522678e-02

1.280000e-08	2.515202e-02

1.320000e-08	2.510190e-02

1.360000e-08	2.506831e-02

1.400000e-08	2.504579e-02

1.440000e-08	2.503069e-02

1.480000e-08	2.502057e-02

1.520000e-08	2.501379e-02

1.560000e-08	2.500924e-02

1.600000e-08	2.500620e-02

1.640000e-08	2.500415e-02

1.680000e-08	2.500278e-02

1.720000e-08	2.500187e-02

1.760000e-08	2.500125e-02

1.800000e-08	2.500084e-02

1.840000e-08	2.500056e-02

1.880000e-08	2.500038e-02

1.920000e-08	2.500025e-02

1.960000e-08	2.500017e-02

2.000000e-08	2.500011e-02

2.040000e-08	2.500008e-02

2.080000e-08	2.500005e-02

2.120000e-08	2.500003e-02

2.160000e-08	2.500002e-02

2.200000e-08	2.500002e-02

2.240000e-08	2.500001e-02

2.280000e-08	2.500001e-02

2.320000e-08	2.500000e-02

2.360000e-08	2.500000e-02

2.400000e-08	2.500000e-02

2.440000e-08	2.500000e-02

2.480000e-08	2.500000e-02

2.520000e-08	2.500000e-02

2.560000e-08	2.500000e-02

2.600000e-08	2.500000e-02

2.640000e-08	2.500000e-02

2.680000e-08	2.500000e-02

2.720000e-08	2.500000e-02

2.760000e-08	2.500000e-02

2.800000e-08	2.500000e-02

2.840000e-08	2.500000e-02

2.880000e-08	2.500000e-02

2.920000e-08	2.500000e-02

2.960000e-08	2.500000e-02

3.000000e-08	2.500000e-02

3.040000e-08	2.500000e-02

3.080000e-08	2.500000e-02

3.120000e-08	2.500000e-02

3.160000e-08	2.500000e-02

3.200000e-08	2.500000e-02

3.240000e-08	2.500000e-02

3.280000e-08	2.500000e-02

3.320000e-08	2.500000e-02

3.360000e-08	2.500000e-02

3.400000e-08	2.500000e-02

3.440000e-08	2.500000e-02

3.480000e-08	2.500000e-02

3.520000e-08	2.500000e-02

3.560000e-08	2.500000e-02

3.600000e-08	2.500000e-02

3.640000e-08	2.500000e-02

3.680000e-08	2.500000e-02

3.720000e-08	2.500000e-02

3.760000e-08	2.500000e-02

3.800000e-08	2.500000e-02

3.840000e-08	2.500000e-02

3.880000e-08	2.500000e-02

3.920000e-08	2.500000e-02

3.960000e-08	2.500000e-02

4.000000e-08	2.500000e-02